AQA GCSE PHYSICS  ·  PAPER 1  ·  FOUNDATION & HIGHER

Exam Practice: Circuit symbols, charge and current

Circuit symbols, charge and current   ·   Electricity   ·   Lesson 1 of 10   ·   14 marks   ·   15 minutes

| Name |  | Date |  |
| --- | --- | --- | --- |

Instructions

•  Answer all the questions.

•  Write your answers in the spaces provided.

•  The marks for each question are shown in brackets - use this as a guide to how much to write.

•  The answers are on separate pages at the back. Attempt every question before you look at them.

•  Answer all questions. Use the mark allocation as a guide to how much to write.

Question 1   CALCULATE	(2 marks)

A current of 0.40 A flows through a lamp for 30 s. Calculate the charge that flows through the lamp. Use the equation: charge flow = current × time

(Total for Question 1 = 2 marks)

Question 2   DRAW	(4 marks)

Draw a circuit diagram to show a cell, a switch and a lamp connected in series, with an ammeter to measure the current through the lamp and a voltmeter to measure the potential difference across the lamp. Use the correct circuit symbols.

(Total for Question 2 = 4 marks)

Question 3   CALCULATE	(3 marks)

180 C of charge flows through a heater in 1.0 minute. Calculate the current in the heater.

(Total for Question 3 = 3 marks)

Question 4   EXPLAIN	(2 marks)

In a series circuit, ammeter A1 reads 0.50 A. A second ammeter, A2, is on the other side of a lamp. What does A2 read? Explain your answer.

(Total for Question 4 = 2 marks)

Question 5   CALCULATE	(3 marks)

A charge of 12 C flows through a resistor when the current is 0.50 A. Calculate the time for which the current flows.

(Total for Question 5 = 3 marks)

TOTAL FOR PAPER = 14 MARKS

Answers and mark scheme

Check your answer only once you have written one.

Question 1	(2 marks)

Q = 0.40 × 30 = 12 C

•   Correct substitution   1 mark

•   12 C   1 mark

Question 2	(4 marks)

A single closed loop containing a cell, a switch, a lamp and an ammeter, with a voltmeter connected in parallel across the lamp only.

•   Cell, switch and lamp in a closed series loop   1 mark

•   Ammeter in series   1 mark

•   Voltmeter in parallel with the lamp   1 mark

•   All symbols correct   1 mark

Question 3	(3 marks)

t = 60 s; I = Q ÷ t = 180 ÷ 60 = 3.0 A

•   Converts 1.0 minute to 60 s   1 mark

•   Correct substitution   1 mark

•   3.0 A   1 mark

Question 4	(2 marks)

0.50 A. The current has the same value at every point in a single closed loop; the lamp does not use up the current.

•   0.50 A   1 mark

•   Current is the same at every point in a series loop   1 mark

Question 5	(3 marks)

t = Q ÷ I = 12 ÷ 0.50 = 24 s

•   Rearranges to t = Q ÷ I   1 mark

•   Correct substitution   1 mark

•   24 s   1 mark